# Ethan Blagdon 20552776

# 2 Axis Robot Writer Project Planning

# Outline of the Problem to be Solved

The code writes and receives data from the Arduino to a 2 axis robot which writes text on a page. The Robot "writes" from G code inputs sent by the Arduino. 1 word is processed at a time and when the robot is ready to receive the next word it sends data to the Arduino requesting more data.

The Code initially opens and reads a txt file containing the font data and gets a user input for the font size. Using the font G code data and size the positions and code for each character in the txt file is translated to the correct format and scaled, before being stored in an array (WordGCode()). Each letter can be accessed from x position in the array, where x is the starting location for each character which is determined by a for loop and the positions are stored in a different array (CharLocation()).

The txt file containing the text to be "written" is then opened, and each word is separated and stored in separate columns of an array  (WordContents()). The initial conditions for the robot are then generated and sent (spindle on, pen up and position at 0,0).

For each word,

1. The x and y starting positions determined (if the word length + current words in line are greater than the maximum x space on page a new line is made and the word starts on the new line (change in y)) (x position is then determined as if its on a new line it is 0 and if its carrying on from the last word it is the ending x point of the last word plus a gap)

2. For each letter

a. The character is read and the location of the G code in WordGCode() is found

b. From the x and y starting point the Gcode is loaded in from the file and the x and y starting positions are added to all lines of the code for that character.

c. The data is formatted into the exact G code style needed and stored in        CurrentWordGCode()

d. The x starting point adds on the width of the character processed.

e. Pen is set to up

3. For all but the first word, the code waits until /n is sent from the robot to request a new words data.

4. The CurrentWordGCode() data is processed into the correct data channels and sent to the robot.

After all words are complete and the text file has reached the end, the pen is set to the up position and is sent to 0,0. Then turn off the robot.

Then end the program.

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| Arrays |  |  |
| WordContents() | Char Array | Breaks down the words in the text to individual characters, then stores each word on a separate column so the code has easy access to which character it needs to process. |
| CharGCode() | Double Array | Contains the converted G code (after font scale is applied) for every letter from start to bottom, with the X,Y and P positions in separate columns. |
| CharLocation() | Integer Array | Contains the starting locations for each character in the CharGCode() array for easy lookup for every character |
| CurrentWordGCode() | Double Array | Holds all the GCode for each character of the current word that is being sent to the robot which will later be converted to serial and sent |
| For then Pen control |  |  |
| Xpos | Integer | X position of the pen for each section of Gcode |
| Ypos | Integer | Y position of the pen for each section of Gcode |
| Ppos | Boolean | 1 or 0 for whether pen is up or down |
| Xspacing | Integer | The set distance to add from the end of the last word |
| Yspacing | Integer | The set y distance between rows of words |
| Penspeed | Integer | The speed of the “spindle” for the robot |
| Pen | Integer | States current position of pen (1 is up and 0 is down) used for sending G code to remove unnecessary callouts to lower / raise pen when it is already in that state |
| For word formatting |  |  |
| NumWords | Integer | Number of words in word contents txt file used for for loops |
| NumLetters | Integer | Numbers of letters in the word used for formatting word position on the paper and for loops |
| WordLength | Integer | Word length derived from number of letters which is used to determine word position on the paper. |
| FontSize | Integer | User inputted font size to alter the GCode to size |
| NumFont | Integer | Number of lines in font txt file used for for loops |
| Miscellaneous |  |  |
| SerialSend | String | Contains the exact line of GCode to be send via serial port. Used in Send to serial port function |

# Function Declarations

Only include functions that you will develop.

Int GetCharLocation( int NumFont, file *Font.txt, int *CharLocation)

Parameters:

NumFont – inputs the length of the Font.txt used in for loop

Font.txt- inputs the font text document to be processed

CharLocation – Pointer to the output array which states the locations each character starst at in CharGCode()

Return value – returns 1 is successful, 0 if failed

Int FormatAndScaleFontData( int NumFont, file *Font.txt, int FontSize, int *CharGCode())

Parameters:

NumFont – inputs the length of the Font.txt used in for loop

Font.txt- inputs the font text document to be processed

FontSize – inputs the FontSize determined by the user

CharGCode() – pointer to the output array which contains the GCode for each character

Return value – returns 1 is successful, 0 if failed

Int SeperateWords(  file *SingleStrokeFont.txt, Char *WordContents() )

Parameters:

Words.txt – inputs the text file that contains the words to be drawn

WordContents() – Pointer to the output array which has separated all characters for each word in array.

Return value – returns 1 is successful, 0 if failed

Int WordToGCodeandSend( char *WordContents(), double *CharGCode(), int *CharLocation())

Parameters:

WordContents() – inputs the array that contains separated words and characters

CharGCode() – inputs the array containing the GCode for each character

CharLocation() – inputs the array containing the location of each character within  CharGCode()

Return value – returns 1 is successful, 0 if failed

Int SendToRobot( Double CurrentWordGCode())

CurrentWordGCode() – inputs the current chunk of instructions to be converted to serial and sent to robot

Return value – returns 1 is successful, 0 if failed

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| Main() |  |  |  |
| GetCharLocation() |  |  |  |
| FormatAndScaleFontData() |  |  |  |
| SeperateWords() |  |  |  |
| WordToGCodeandSend() |  |  |  |
| SendToRobot() |  | CurentWordGCode() | Robot receives all data and “writes” the GCode in CurentWordGCode() |

Extend table as required. Note that ‘Function’ includes main()

# Flowchart(s)

These are in separate PDF’s

# AI Statement